Helios T-5 and T-5 II High Performance

Complete Fluorescent Light Fixtures

Read and follow the guidelines to insure the proper use and application for this Helios light fixture. Failure to follow these guidelines may result in light fixture damage, electrical shock, fire and/or injury.

Important warning and safeguards

- Read and follows all instructions.

- The national code requires all aquarium equipment to be plugged into a GFCI (Ground Fault Circuit Interrupter) electrical outlet.  The national code requires that a GFCI (ground fault circuit interrupter) be used in the branch circuit supplying all water pumps and electrical aquarium equipment. If you do not have a GFCI have an electrician install prior to operating any aquarium component.

- Do not use with an extension cord or power strip.

- Use this product for its intended use only.

- Never yank or pull the cord from the electrical outlet.

- Do not operate if the plug or wire is damaged. Disregard unit.

- The light fixture is not to come in contact with water.

- The light fixture must be turned off/disconnect from the power supply before weekly maintenance is done to either the light fixture or the aquarium.

- Verify that hands are dry prior to disconnecting the power.

- To reduce the risk of electrocution keep all connections dry.

- If the light fixture is to fall or come in contact with water/excess moisture unplug be light fixture from the power supply before removing. Failure to do so may cause electrical shock or injury.

- At all times the light fixture and connections need to be free of water, moisture and salt build up. Failure to do so will cause electrical arching resulting in fire or electrical shock.

- Do not place above an open top aquarium.

- Do not use flexible jumpers (20847) with HO light fixtures.

It is your sole responsibility to verify that the plug and the receptacle are clean and free of moisture and salt build up at all times. The receptacle must be free of water, salt, calcium and magnesium failure to do so may cause a fire and/or permanent damage to the light fixture. Do not plug into an extension cord.

The user must arrange a drip loop if the pump is plugged in to a GFCI outlet below the aquarium. A drip loop is that part of the cord hanging below the receptacle.

Drip Loop

Instructions for installation T-5 light fixtures

1.  Mounting of light fixtures

  - Use the mounting brackets and screws to secure the light inside a canopy.

  - Snap light fixture into the mounting brackets.

  - Verify that the light fixture is secure

  - Plug power supply cord into the light fixture

  - Plug cord directly into a G.F.C.I. receptacle.

- Installation of flexible jumpers- Regular output T-5

  - Do not daisy chain more lights than recommended

  - Plug power supply cord into the light fixture.

  - With a small screwdriver remove the cap that covers the plug socket of the light fixture. Located on the opposite side of where the power supply cord connects.

  - Now connect the two light fixtures with the Flexible Jumper.

  - If more lights need to be daisy chained repeat step c and d.

| Wattage | Size | Maximum number |
| --- | --- | --- |
| 13 watt | 19" | 14 |
| 18 watt | 23" | 10 |
| 24 watt | 29" | 8 |
| 28 watt | 35" | 7 |
| 38 watt | 47' | 5 |
| 48 watt | 59" | 4 |

Instructions for installation T-5 High Output light fixtures:

Do not use flexible jumpers with HO fixtures

1.  Mounting of light fixtures: 23”,35” and 47”

- Use the mounting brackets and screws to secure the light inside a canopy.

- Snap light fixture into the mounting brackets.

- Verify that the light fixture is secure

- Plug power supply cord into the light fixture

- Plug cord directly into a G.F.C.I. receptacle.

2.  Mounting of light fixtures: 59” and 71” HO units require an remote electronic ballast

  - Use the mounting brackets and screws to secure the light inside a canopy.

  - Snap light fixture into the mounting brackets.

  - Verify that the light fixture is secure

  - Mount ballast in a well-ventilated dry area.

  - Plug the ballast into the light fixture.

  - Plug ballast directly into a G.F.C.I. receptacle.

Instructions for installation T-5 II light fixtures

1.  Mounting of light fixtures

- Use the mounting brackets and screws to secure the light inside a canopy.

- Snap light fixture into the mounting brackets.

- Verify that the light fixture is secure

- Plug power supply cord into the light fixture

- Plug unit directly into a G.F.C.I. receptacle.

2.        Installation of coupler- regular output T-5 II

- Do not daisy chain more lights than recommended

- Connect the T coupler to the pigtail on the T-5 II lamp

- To connect more than two units connect the two T couplers together using the two open ends will connect directly two the T-5 II lamps.

- For more T-5 II light fixtures repeat step C.

- Plug the power supply cord into the coupler.

- Secure the coupler and all connections so they will not come in contact with water, moisture and salt deposits.

- Plug the cord directly into a G.F.C.I. receptacle

| Wattage | Size | Maximum number |
| --- | --- | --- |
| 13 watt | 19" | 14 |
| 18 watt | 23" | 10 |
| 24 watt | 29" | 8 |
| 28 watt] | 35" | 7 |
| 38 watt | 47' | 5 |
| 48 watt | 59" | 4 |

Instructions for installation T-5 II High Output light fixtures:

Do not use couplers with HO fixtures

1.  Mounting of light fixtures: 23”, 35” and 47”

a. Use the mounting brackets and screws to secure the light inside a canopy.

b. Snap light fixture into the mounting brackets.

- Verify that the light fixture is secure

- Plug power supply cord into the light fixture

- Plug cord directly into a G.F.C.I. receptacle.

2.  Mounting of light fixtures: 59” and 71” HO units require remote electronic ballast

  - Use the mounting brackets and screws to secure the light inside a canopy.

  - Snap light fixture into the mounting brackets.

  - Verify that the light fixture is secure

  - Mount ballast in a well-ventilated dry area.

  - Plug the ballast into the light fixture.

  - Plug ballast directly into a G.F.C.I. receptacle

Instructions for installation of Store Use T-5 dual light fixtures 2 options

1. Mounting of light fixtures

- Use the mounting brackets and screws to secure the light inside a canopy.

- Snap light fixture into the mounting brackets.

- Verify that the light fixture is secure

- Plug power supply cord into the light fixture

- Plug cord directly into a G.F.C.I. receptacle.

- Hanging of light fixture: Hangers are to sold separately

## Limited Warranty

Commodity Axis Inc. warrants that the Helios T-5 light fixture is free of defects for a period of six months from the initial date of purchase shown with the original cash register receipt. Commodity Axis Inc. disclaims all other warranties of merchantability and/or fitness for a particular purpose. Although this limited warranty may give you specific rights, you may have other rights that may vary from state to state. This warranty is void if the failure of the Helios T-5 light fixture or any part, sealant or component there of is due to (i) misuse, (ii) tampering, (iii) negligence, (iv) misapplication, (v) abuse, (vi) accident and/or (vii) failure to properly (a) maintain, (b) clean, (c) keep dry and free of salt buildup and or use the Helios T-5 light fixture with a Ground Fault Circuit Interrupter (GFCI).

# Consumer Remedies

Commodity Axis Inc. entire liability and your sole exclusive remedy be, either the repair or replacement of the Helios T-5 light fixture only if the return the defective Helios T-5 light fixture directly to Commodity Axis Inc. with the original cash register receipt in this original box along with $7.00 for shipping and handling. Do not send the Helios T-5 light fixture by insured mail. The Helios T-5 light fixture may be sent by UPS requiring a signature on arrival. Any replacement of Helios T-5 light fixture will be warranted for the remaining for the original warranty period. In no event will shipping charges be reimbursed.

In not event will Commodity Axis Inc. be liable for any consequential or incidental damages including, but not limited to loss of life, personal injury, property damage, damaged equipment, lost profits, lost wages, lost savings, lost income, etc. Some states do not allow for the limitations or exclusion of liability for incidental or consequential damages, so the above exclusion may not apply to you.